JS-20 „LARGO“ P.I.R. detector

Ce détecteur de mouvement P.I.R. est utilisé pour protéger les intérieurs. Il détecte les mouvements d’objets à une température proche de celle du corps humain.

Le signal du capteur est analysé de façon électronique. Cela afin d’assurer une excellente sensibilité et d’éradiquer en même temps les fausses alertes.

Le taux d’analyse de détection peut être ajusté afin d’augmenter son immunité si le JS- 20 est installé dans un endroit problématique.

Dans le détecteur, la lentille standard peut être remplacée par un couloir  facultatif ou une lentille inviolable.  Des terminaux supplémentaires  (GBS) peuvent être utilisés pour faciliter le câblage du détecteur.

Le JS-20 se distingue par son excellente immunité RF

Il peut être installé sur un mur plat ou dans un coin.

# Specification

Power supply:	12 V DC ± 25%

Power consumption (LED off):	max. 10 mA

Maximum consumption (LED on):	max. 35 mA

Terminals size:	max. 1 mm2

Mounting height:	2.5 m above floor

Detection range:	120° / 12 m (standard lens)

Initialization:	aprox. 1 minute

Alarm output:	normally closed, max. 60V / 50 mA,

Internal resistance max.30 Ohm

Tamper output:	normally closed, max. 60 V / 50 mA,

Internal resistance max.16 Ohm

Environment 	II. – General indoor, (EN 50131-1)

Operating temperatures	-10 to +55 °C

Security level	grade 2, EN 50131-1

Complies with the essential requirements of: 89/336/EC EMC Directive  - Protection concerning electromagnetic compatibility when is used for its intended purpose. Original of the conformity assessment can be found at the web page section Technical support.

# Installation

Ce détecteur est destiné à un usage intérieur.

Il peut être installé sur un mur plat ou dans un coin. Ne pas placer le détecteur près d’une source de chaleur ou d’un climatiseur ou près  d’un objet  dont la température varie souvent.

Le détecteur doit avoir une vue dégagée de la zone à protéger.

- Ouvrir le couvercle du détecteur (appuyer avec un tournevis sur l’étiquette en plastique de la partie du fond du logement du détecteur).

- Enlever le PCB en appuyant sur l’étiquette en plastique souple à l’intérieur du logement.

- Enfoncer les trous préformés pour les câbles et les vis.

- Attach the housing to the wall (2.5 meters above the floor).

- Return the PCB into the rear housing and connect the cable wires to the terminals.

- Attach the cover.

Note: Avoiding touching or scratching the PIR sensor!

## Terminals

+12V, GND	power supply

PIR, PIR	PIR alarm output (normally closed)

TMP, TMP	Tamper output (normally closed)

GBS, GBS	auxiliary terminals – useful when wiring (see examples)

#### Jumpers

LED 	the LED indicator can be disabled by opening this jumper

DETECTION 	by opening this jumper you can set a higher rate of analysis and increase the immunity of the detector. This setting is suitable for problematic locations with sudden temperatures changes or electromagnetic interference. Reaction of the detector will be slightly slower in this setting.

### Detector testing

- After switching on the power, wait for 1 minute. During this period the detector initializes and stabilizes its parameters (LED will be on if not disabled).

- Each detected movement will be indicated by the detector’s LED (if not disabled)

- Confirm that the detector covers the protected area as you expected.

### Lenses

Standard lens that is supplied with the JS-20 Largo detector covers an area of 120 degrees / 12 meters.

For special applications other lenses can be used. Optional lenses for long corridors and areas with small animals are supplied separately.

Pet lens is marked JS-7906. This lens prevents animals smaller then 40 cm to be detected by the detector. The working range of this lens is reduced to 7 meters. It is recommended to test this lens carefully with the animal when installing the system, in order to prevent false alarms.

Corridor lens is marked JS-7904. The working range of this lens is 25 meters long and only 3 meters wide. It is recommended to test the detector’s sensitivity carefully when installing it into a corridor.

Note: Although this product does not contain any harmful materials we suggest you to return the product to the dealer or directly to the producer after usage.

Examples of wiring the JS-20 Largo detector using end-of-line resistors and double end-of-line resistors

(Auxiliary GBS terminals are used to facilitate the wiring)

Example of wiring of several JS-20 Largo detectors into the double EOL loop

(Auxiliary GBS terminals are used to facilitate wiring)

Example of wiring of several JS-20 Largo detectors into the EOL loop

(Auxiliary GBS terminals are used to facilitate the wiring)